| Bandalux Industrial S.A. Av. de Sant Julià, 235 08403 Granollers, Barcelona. Spain. | Phone: +34 93 861 51 79 Website: www.contract.bandalux.com contract.international@bandalux.com |
| --- | --- |

Z-BOX SECRET SPECIFICATIONS

Instructions for use:

Paragraphs with blue text between [ ] are optional elements that require selection by the specifier. For example “Colour: [white] [black]”. Once the selections are made and other options are removed, the blue font colour should be changed to black.

Texts in green between ** are the fabrics that should be added by the specifier.

We recommend checking with our team of specifiers to ensure the system’s compatibility with the fabric and type of operating system.

Z-BOX SECRET roller blind with cassette for [INTERIOR] [EXTERIOR]

System Bandalux Z-BOX SECRET roller blind with cassette and bottom access for maintenance in a system that is fully integrated into the facade. Class 6 wind resistance according to EN 13561. [Interior] [Exterior] installation via [frontal] [lateral] fixed Zip Secret side channels. [When the system is combined with a blackout fabric it blocks out sunlight]. End pieces with calibrated stainless steel rods that rotate on encapsulated stainless steel bearings for smooth movement and noise reduction (no maintenance required). System with class 3 durability according to EN13561: 10,000 extensions and 10,000 retractions. The fabric is rolled onto a galvanised steel tube designed to prevent marks from forming on the fabric. Aluminium and AiSi 316 stainless steel metal components with anti-corrosion coating to ensure compliance with EN13561, Class 4. The 122x160mm cassette is formed by extruded aluminium profiles with [Qualicoat lacquer] [anodised Qualanod] finish. It includes an IP65 rated cable grommet that prevents water from entering the motor headrail through the cable. The bottom cover can be opened to allow access to the inside without having to completely take the system apart. The Max terminal is concealed in the cassette when the curtain is retracted. Cassette, side channels, accessories and hembar in the same matching RAL colour: [white RAL 9016] [black RAL 9011] [anthracite RAL 7016] [special RAL]. 3-year warranty. Operating system Operating system: [Somfy hardwired] [Somfy RTS radio] [Somfy Silent RTS radio] [Bandalux radio] [Somfy IO radio] motor with a Bandalux designed and patented drive wheel that reduces the noise and vibration the motor makes when turning. Fabric *Insert the details of the chosen fabric here: https://contract.bandalux.com/en/fabrics*.